Assignment No: 3

------------------------------------------------------------------------------------------------

Implementing a real-time undo/redo system for a text editing application using a Stack data structure. The system should support the following operations:

class TextEditor:

def __init__(self):

self.document = []           # List to store each line of the document

self.undo_stack = []         # Stack to store previous states for Undo

self.redo_stack = []         # Stack to store undone states for Redo

# Add a new line/text to the document

def make_change(self, change):

self.undo_stack.append(self.document.copy())  # Save current state for Undo

self.document.append(change)                  # Add new line/text

self.redo_stack.clear()                        # Clear Redo stack after a new change

print(f"\nChange Made: '{change}'")

# Undo the last change

def undo(self):

if not self.undo_stack:

print("\nNothing to Undo!")

return

self.redo_stack.append(self.document.copy())  # Save current state for Redo

self.document = self.undo_stack.pop()        # Revert to the previous state

print("\nUndo performed.")

# Redo the last undone change

def redo(self):

if not self.redo_stack:

print("\nNothing to Redo!")

return

self.undo_stack.append(self.document.copy())       # Save current state for Undo

self.document = self.redo_stack.pop()                      # Reapply the undone change

print("\nRedo performed.")

# Display the current document

def display_state(self):

print("\n--- Current Document ---")

if not self.document:

print("[Empty Document]")

else:

for i in range(len(self.document)):    # loop over index numbers

print(f"{i+1}: {self.document[i]}")   # i+1 for line number, document[i] for text

print("------------------------")

# ----------------- Menu Driven Code -----------------

editor = TextEditor()

while True:

# Display menu

print("\n--- Menu ---")

print("1. Add Line/Text")

print("2. Undo")

print("3. Redo")

print("4. Show Document")

print("5. Exit")

choice = input("Enter your choice: ")

if choice == '1':

text = input("Enter line/text to add: ")

editor.make_change(text)  # Add new line/text

elif choice == '2':

editor.undo()             # Undo last change

elif choice == '3':

editor.redo()             # Redo last undone change

elif choice == '4':

editor.display_state()    # Display current document

elif choice == '5':

print("Exiting...")

break

else:

print("Invalid choice! Try again.")

Output:-

--- Menu ---

1. Add Line/Text

2. Undo

3. Redo

4. Show Document

5. Exit

Enter your choice: 1

Enter line/text to add: neha

Change Made: 'neha'

--- Menu ---

1. Add Line/Text

2. Undo

3. Redo

4. Show Document

5. Exit

Enter your choice: 2

Undo performed.

--- Menu ---

1. Add Line/Text

2. Undo

3. Redo

4. Show Document

5. Exit